ETIN 40

IC project 2

Final Report

Hanyu Liu

February 27, 2023

Abstract

In this report, we verified the hardware accelerator which was designed in ICP 1. The SRAM which was used in ICP 1 was replaced by distributed memory generator with Xilinx IP. The input data and coefficients will be embedded into the memory as coe file. Therefore, the finite state machine needs to be re-designed to fulfill the new requirements. The verification will be completed by Vivado and a logic analyzer.

Contents

1. Introduction	4

2. Implementation	5

2.1 Fixed block diagram	5

2.2 Fixed ASMD charts	6

2.2.1 Controller	6

2.2.2 Load coefficient	8

2.2.3 Load input	9

3. Verification	11

4. Appendix	15

4.1 VHDL code for controller	15

4.2 VHDL code for load_coeff	18

4.3 VHDL code for load_input	21

4.4 VHDL code for multiply	27

4.5 VHDL code for store	32

4.6 VHDL code for max	34

4.7 VHDL code for top	36

# 1. Introduction

The project aims to verify the IC project 1 hardware accelerator that realizes the multiplication of an input matrix with a size of 14x8 and a coefficient matrix with a size of 8x14. The input data and coefficients are pre-stored in SRAMs with Xilinx IP, respectively. And the results will be stored in another SRAM after calculation. The hardware accelerator is constructed by using VHDL language and verified by a logic analyzer.

To verify the design in IC project 2, the state machine has been changed in modules “controller”, “load_coeff” and “load_input”. The block diagram has also been changed accordingly which has shown in the following paragraph. All the verifications are based on the FPGA of Xilinx “xc7a100tcsg324-1”.

# 2. Implementation

In this part, the fixed block diagram and fixed ASMD charts will be introduced respectively. Comparing the ASMD charts in IC project 1, some states were deleted to adapt to the new requirements in IC project 2. A different design will be introduced in this part.

## 2.1 Fixed block diagram

The block diagram of the top module is shown in Figure 2.1. This matrix multiplier can be divided into 9 modules, which are “load_coeff”, “load_input”, “multiply”, “store”, “max”, “controller”, and three SRAMs. There are two inputs and one output for the top module to adapt the new design in IC project 2, “start”, “restart” and “max_out”.

Figure 2.1: The block diagram of the matrix multiplier

In IC project 2, “SRAM_coe” and “SRAM_input”, two SRAMs keep the coefficient matrix and input matrix respectively. Module “load_input” will generate the address for “SRAM_input” and then temporarily keep for a row of input matrix by four 16-bit registers. Module “load_coeff” will generate an address for “SRAM_coe” and send the data directly to the module “multiply”.

## 2.2 Fixed ASMD charts

To adapt the IC project 2, there are three fixed ASMD charts shown in this part, “controller”, “load_coeff”, and “load_input”. In this part, the fixed state machine will be introduced.

### 2.2.1 Controller

The ASMD chart of the “controller” is shown in Figure 2.2.1. There are three states in this module.

Figure 2.2.1: The ASMD chart of the “controller”

S_initial:

In this state, all signals are set to default values.  Signal “flag1” in IC project 1 has been deleted due to all the data has been stored in SRAMs. There is no need to get the data from the outside. Thus, once the signal “start” is HIGH, the calculation will start immediately.

S_ld_input:

In this state, 8 inputs will be loaded from “SRAM_input” and stored in four 16-bit registers. After that, the signal “ldinput_done” will be set to HIGH, which indicates that the system is ready to produce a column of results.

S_op:

The state “s_op” is where the multiplication and accumulation take place. There is no change compared with the design in IC project 1.

### 2.2.2 Load coefficient

The ASMD chart of “load_coeff” is shown in Figure 2.2.2. There are three states in this module.

Figure 2.2.2: The ASMD chart of “load_coefficient”

In the IC project 1 design, there are four states in module “load_coeff”. There is no need to load the coefficients from outside into SRAM. All the data has been embedded by coe file. However, the other three states kept making the state machine work well.

### 2.2.3 Load input

The ASMD chart of “load_input” is shown in Figure 2.2.3. There are five states in this module.

Figure 2.2.3: The ASMD chart of “load_input”

S_initial:

In this state, all variables will be set to default values. There is no change compared to the design with it in IC project 1.

S_ld_input_1:

This is the new state in IC project 2. In this state, the module will send the address to the “SRAM_input”. After receiving one raw input data, the state machine will return to the initial state.

S_ld_input_2:

This is also the new state in IC project 2. In this state, the system will receive the input data from the SRAM and store them in registers. Then the system will get back to state “S_ld_input_1” to do the next execution.

S_send2multi_w1:

This state will wait for a clock cycle to ensure that the coefficients have been received.

S_send2multi:

In this state, 8 inputs will be sent to the module “multiply” two by two.

# 3. Verification

The multiplication was verified in Vivado, and the simulation results are shown in Figures 3.1 and 3.2. The results are stored in SRAM in order successfully which has been shown in figure 3.3. Figure 3.4 shows the system iterated 14 times due to 14 rows in the input matrix. Figure 3.5 shows the data are the same in both the simulation and logic analyzer.

store signal

Result from each multiplier

input

coefficient

1336

812

554

114

480

44

120

11

4

4

189

69

63

23

3

3

204

102

118

2

2

236

53

61

53

61

1

1

2

10

28

60

Figure 3.1: Simulation result from Vivado

794

794

344

88

86

22

4

4

3

3

362

63

21

39

13

260

126

88

46

44

23

2

51

75

51

75

1

2

1

Figure 3.2: Another group of simulation results from Vivado

556

1136

384

Figure 3.3: The results in decimal stored in memory successfully.

14

13

12

11

10

9

8

7

6

5

4

3

2

1

Figure 3.4: The calculation will iterate 14 times due to 14 rows in the input matrix.

0

0

0

1

1

0

1

1

1

0

0

0

Figure 3.5: Compare the data detected in the logic analyzer with the simulation result.

# 4. Appendix

## 4.1 VHDL code for controller

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity controller is

Port (

clk, reset  : in std_logic;

start       : in std_logic;

ld_input_done   : in std_logic;

multi_done  : in std_logic;

ld_input    : out std_logic;

op_en       : out std_logic

);

end controller;

architecture Behavioral of controller is

component ff is

generic(N:integer:=1);

port(   D  :  in std_logic_vector(N-1 downto 0);

Q  :  out std_logic_vector(N-1 downto 0);

clk  :  in std_logic;

reset:  in std_logic

);

end component;

type state_type is (s_initial, s_ld_input, s_op);

signal state_reg, state_nxt : state_type;

signal counter14, counter14_nxt : std_logic_vector(3 downto 0) := (others => '0');

begin

--state contrl--------------------------------

process(clk, reset)

begin

if reset = '1' then

state_reg <= s_initial;

elsif (clk'event and clk = '1') then

state_reg <= state_nxt;

end if;

end process;

--state machine--------------------------------------------

process(state_reg, start, ld_input_done, multi_done, counter14)

begin

ld_input <= '0';

op_en <= '0';

counter14_nxt <= counter14;

case state_reg is

when s_initial =>

counter14_nxt <= (others => '0');

if start = '1' then

state_nxt <= s_ld_input;

else

state_nxt <= s_initial;

end if;

when s_ld_input =>

ld_input <= '1';

if ld_input_done = '1' then

state_nxt <= s_op;

else

state_nxt <= s_ld_input;

end if;

when s_op =>

if multi_done = '1' then

if counter14 = "1101" then

counter14_nxt <= (others => '0');

state_nxt <= s_initial;

op_en <= '0';

else

counter14_nxt <= counter14 + 1;

state_nxt <= s_ld_input;

op_en <= '0';

end if;

else

op_en <= '1';

counter14_nxt <= counter14;

state_nxt <= s_op;

end if;

end case;

end process;

counter_14: FF

generic map(N => 4)

port map(   D     =>counter14_nxt,

Q     =>counter14,

clk     =>clk,

reset   =>reset

);

end Behavioral;

## 4.2 VHDL code for load_coeff

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity load_coeff is

Port (

clk, reset  : in std_logic;

--signal from controller

op_en       : in std_logic;

--control signal to multiply

multi_en    : out std_logic;

--coeff to multiply

data_coeff  : out std_logic_vector(15 downto 0)

);

end load_coeff;

architecture Behavioral of load_coeff is

component SRAM_coe

port (

clk     : in  std_logic;

we      : in std_logic;

a       : in  std_logic_vector (6 downto 0);

d       : in  std_logic_vector (15 downto 0);

qspo    : out std_logic_vector (15 downto 0)

);

end component;

component ff is

generic(N:integer:=1);

port(   D  :  in std_logic_vector(N-1 downto 0);

Q  :  out std_logic_vector(N-1 downto 0);

clk  :  in std_logic;

reset:  in std_logic

);

end component;

--SRAM---------------------------------------------

signal choose       : std_logic;

signal r_or_w       : std_logic;

signal address      : std_logic_vector(6 downto 0);

---------------------------------------------------

type state_type is (s_initial, s_op, s_send2multi);

signal state_reg, state_nxt : state_type;

signal counter_1, counter_1_nxt : std_logic_vector(5 downto 0) := (others => '0');

signal coeff_32 : std_logic_vector(15 downto 0);

signal data_coeff_32 : std_logic_vector(15 downto 0);

signal coeff       : std_logic_vector(15 downto 0);

begin

--SRAM bits transfer----------------------

coeff_32 <= coeff;

data_coeff <= data_coeff_32(15 downto 0);

------------------------------------------

Ram_coeff: SRAM_coe

port map(

clk     => clk,

we      => r_or_w,

a       => address,

d       => coeff_32,

qspo    => data_coeff_32

);

--state contrl--------------------------------

process(clk, reset)

begin

if reset = '1' then

state_reg <= s_initial;

elsif (clk'event and clk = '1') then

state_reg <= state_nxt;

end if;

end process;

--state machine--------------------------------------------

process(state_reg, op_en, counter_1)

begin

--SRAM------------------------------

choose <= '1';

r_or_w <= '0';--read

address <= "0" & counter_1;

------------------------------------

counter_1_nxt <= (others => '0');

multi_en <= '0';

case state_reg is

when s_initial =>

if op_en = '1' then

state_nxt <= s_op;

else

state_nxt <= s_initial;

end if;

when s_op =>

choose <= '0';

r_or_w <= '0'; --read

address <= "0" & counter_1;

counter_1_nxt <= counter_1;

if counter_1 = "111000" then --counter = 56 (address = 0 -55)

state_nxt <= s_initial;

elsif counter_1 = "000000" then

multi_en <= '1';

state_nxt <= s_send2multi;

else

multi_en <= '0';

state_nxt <= s_send2multi;

end if;

when s_send2multi =>

choose <= '0';

r_or_w <= '0'; --read

address <= "0" & counter_1;

counter_1_nxt <= counter_1 + 1;

state_nxt <= s_op;

end case;

end process;

counter1: FF

generic map(N => 6)

port map(   D     =>counter_1_nxt,

Q     =>counter_1,

clk     =>clk,

reset   =>reset

);

end Behavioral;

## 4.3 VHDL code for load_input

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity load_input is

Port (

clk, reset  : in std_logic;

-----------------------------------------------------

ld_input    : in std_logic;

ld_input_done   : out std_logic;--feedback to controller

--signal from controller

op_en       : in std_logic;

start_ld_input  : out std_logic;

data_input  : out std_logic_vector(15 downto 0)

);

end load_input;

architecture Behavioral of load_input is

component SRAM_input

port (

clk     : in  std_logic;            --Active Low

we      : in std_logic;

a       : in  std_logic_vector (6 downto 0);

d       : in  std_logic_vector (15 downto 0);

qspo    : out std_logic_vector (15 downto 0)

);

end component;

component ff is

generic(N:integer:=1);

port(   D  :  in std_logic_vector(N-1 downto 0);

Q  :  out std_logic_vector(N-1 downto 0);

clk  :  in std_logic;

reset:  in std_logic

);

end component;

--SRAM---------------------------------------------

signal choose       : std_logic;

signal r_or_w       : std_logic; -- Active Low (reand & write) --write '0' --read '1'

signal address      : std_logic_vector(6 downto 0);

---------------------------------------------------

type state_type is (s_initial, s_ld_input_1, s_ld_input_2, s_send2multi, s_send2multi_w1);

signal state_reg, state_nxt : state_type;

signal reg_1, reg_1_nxt : std_logic_vector(15 downto 0);

signal reg_2, reg_2_nxt : std_logic_vector(15 downto 0);

signal reg_3, reg_3_nxt : std_logic_vector(15 downto 0);

signal reg_4, reg_4_nxt : std_logic_vector(15 downto 0);

--enable write in reg

signal flag1 : std_logic;

--enable send data

signal flag2 : std_logic;

signal hold, hold_nxt : std_logic_vector(0 downto 0) := (others => '0');

--control load

signal counter1, counter1_nxt : std_logic_vector(2 downto 0) := (others => '0');

--control send

signal counter2, counter2_nxt : std_logic_vector(1 downto 0) := (others => '0');

--control the loop will be executed 14 times

signal counter3, counter3_nxt : std_logic_vector(3 downto 0) := (others => '0');

signal counter4, counter4_nxt : std_logic_vector(3 downto 0) := (others => '0');

signal input_32 : std_logic_vector(15 downto 0);

signal input    : std_logic_vector(15 downto 0);

begin

Ram_input: SRAM_input

port map(

clk     => clk,

we      => r_or_w,

a       => address,

d       => input_32,

qspo    => input

);

--state contrl----------------------------------------------

process(clk, reset)

begin

if reset = '1' then

state_reg <= s_initial;

elsif (clk'event and clk = '1') then

state_reg <= state_nxt;

end if;

end process;

--state machine --------------------------------------------

process(state_reg, ld_input, op_en, counter1, counter2, counter3, counter4, hold)

begin

start_ld_input <= '0';

ld_input_done <= '0';

counter1_nxt <= (others => '0');

counter2_nxt <= (others => '0');

counter3_nxt <= (others => '0');

counter4_nxt <= counter4;

flag1 <= '0';

flag2 <= '0';

hold_nxt <= (others => '0');

r_or_w <= '0'; --always read

address <= "000" & counter4;

case state_reg is

when s_initial =>

if ld_input = '1' and op_en = '0' then

start_ld_input <= '1';--give signal to outside

state_nxt <= s_ld_input_1;

elsif ld_input = '0' and op_en = '1' then

state_nxt <= s_send2multi_w1;

else

state_nxt <= s_initial;

end if;

when s_ld_input_1 =>

flag1 <= '1';

if counter1 > "011" then

start_ld_input <= '1';

ld_input_done <= '1';

counter1_nxt <= (others => '0');

state_nxt <= s_initial;

else

start_ld_input <= '1';

ld_input_done <= '0';

counter1_nxt <= counter1;

state_nxt <= s_ld_input_2;

end if;

when s_ld_input_2 =>

flag1 <= '1';

counter4_nxt <= counter4 + 1;

counter1_nxt <= counter1 + 1;

state_nxt <= s_ld_input_1;

when s_send2multi_w1 =>

state_nxt <= s_send2multi;

when s_send2multi =>

flag2 <= '1';

if hold = "0" then

hold_nxt <= hold + 1;

state_nxt <= s_send2multi;

counter2_nxt <= counter2;

counter3_nxt <= counter3;

else

hold_nxt <= (others => '0');

if counter3 = "1101" and counter2 = "11" then

counter2_nxt <= (others => '0');

counter3_nxt <= (others => '0');

state_nxt <= s_initial;

elsif counter3 < "1101" and counter2 = "11" then

counter3_nxt <= counter3 + 1;

counter2_nxt <= (others => '0');

state_nxt <= s_send2multi;

else

counter3_nxt <= counter3;

counter2_nxt <= counter2 + 1;

state_nxt <= s_send2multi;

end if;

end if;

end case;

end process;

reg_1_nxt <= input when counter1 = "000" and flag1 = '1' else reg_1;

reg_2_nxt <= input when counter1 = "001" and flag1 = '1' else reg_2;

reg_3_nxt <= input when counter1 = "010" and flag1 = '1' else reg_3;

reg_4_nxt <= input when counter1 = "011" and flag1 = '1' else reg_4;

--Send the data --------------------------------------------

data_input <=   reg_1 when counter2 = "00" and flag2 = '1' else

reg_2 when counter2 = "01" and flag2 = '1' else

reg_3 when counter2 = "10" and flag2 = '1' else

reg_4 when counter2 = "11" and flag2 = '1' else

(others => '0');

--Flip Flop ------------------------------------------------

reg_01: FF

generic map(N => 16)

port map(   D     =>reg_1_nxt,

Q     =>reg_1,

clk     =>clk,

reset   =>reset

);

reg_02: FF

generic map(N => 16)

port map(   D     =>reg_2_nxt,

Q     =>reg_2,

clk     =>clk,

reset   =>reset

);

reg_03: FF

generic map(N => 16)

port map(   D     =>reg_3_nxt,

Q     =>reg_3,

clk     =>clk,

reset   =>reset

);

reg_04: FF

generic map(N => 16)

port map(   D     =>reg_4_nxt,

Q     =>reg_4,

clk     =>clk,

reset   =>reset

);

counter_01: FF

generic map(N => 3)

port map(   D     =>counter1_nxt,

Q     =>counter1,

clk     =>clk,

reset   =>reset

);

counter_02: FF

generic map(N => 2)

port map(   D     =>counter2_nxt,

Q     =>counter2,

clk     =>clk,

reset   =>reset

);

counter_03: FF

generic map(N => 4)

port map(   D     =>counter3_nxt,

Q     =>counter3,

clk     =>clk,

reset   =>reset

);

counter_04: FF

generic map(N => 4)

port map(   D     =>counter4_nxt,

Q     =>counter4,

clk     =>clk,

reset   =>reset

);

hold_time : FF

generic map(N => 1)

port map(   D     =>hold_nxt,

Q     =>hold,

clk     =>clk,

reset   =>reset

);

end Behavioral;

## 4.4 VHDL code for multiply

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity multiply is

Port (

clk, reset  : in std_logic;

-----------------------------------------------------

--signal from load_coeff

multi_en    : in std_logic;

--data in

data_input  : in std_logic_vector(15 downto 0);

data_coeff  : in std_logic_vector(15 downto 0);

-----------------------------------------------------

--ctrl out

multi_done  : out std_logic;

store_en    : out std_logic;

max_en      : out std_logic;

--data out

data_out    : out std_logic_vector(17 downto 0)

-----------------------------------------------------

);

end multiply;

architecture Behavioral of multiply is

component ff is

generic(N:integer:=1);

port(   D  :  in std_logic_vector(N-1 downto 0);

Q  :  out std_logic_vector(N-1 downto 0);

clk  :  in std_logic;

reset:  in std_logic

);

end component;

type state_type is (s_initial, s_multi, s_add, s_send);--, s_wait);

signal state_reg, state_nxt : state_type;

signal input_1, input_2 : std_logic_vector(7 downto 0);

signal coeff_1, coeff_2 : std_logic_vector(6 downto 0);

signal result_1, result_2   : std_logic_vector(17 downto 0);

--the matrix is [14*8]*[8*14], when counting 111 in binary means one number is done.

signal counter_8, counter_8_nxt     : std_logic_vector(1 downto 0) := (others => '0');

signal counter_14, counter_14_nxt   : std_logic_vector(3 downto 0) := (others => '0');

signal counter_6, counter_6_nxt     : std_logic_vector(2 downto 0) := (others => '0');

--------------------------------------------------------------------------------------

signal data, data_nxt   : std_logic_vector(17 downto 0);

signal store, store_nxt : std_logic_vector(0 downto 0);

begin

--state contrl--------------------------------

process(clk, reset)

begin

if reset = '1' then

state_reg <= s_initial;

elsif (clk'event and clk = '1') then

state_reg <= state_nxt;

end if;

end process;

--state machine--------------------------------------------

process(state_reg, multi_en, counter_8, data, result_1, result_2, counter_14, counter_6)

begin

multi_done <= '0';

data_nxt <= (others => '0');

counter_8_nxt <= (others => '0');

counter_14_nxt <= (others => '0');

counter_6_nxt <= (others => '0');

store_nxt <= "0";

input_1 <= (others => '0');

input_2 <= (others => '0');

coeff_1 <= (others => '0');

coeff_2 <= (others => '0');

case state_reg is

when s_initial =>

if multi_en = '1' then

state_nxt <= s_multi;

else

state_nxt <= s_initial;

end if;

when s_multi =>

input_1 <= data_input(7 downto 0);--the input has 8 bits

input_2 <= data_input(15 downto 8);

coeff_1 <= data_coeff(6 downto 0);--the coeff only has 7 bits

coeff_2 <= data_coeff(14 downto 8);

if counter_8 = "00" then

data_nxt <= (others => '0');

else

data_nxt <= data;

end if;

counter_8_nxt <= counter_8;

counter_14_nxt <= counter_14;

state_nxt <= s_add;

when s_add =>

data_nxt <= result_1 + result_2 + data;

input_1 <= data_input(7 downto 0);--the input has 8 bits

input_2 <= data_input(15 downto 8);

coeff_1 <= data_coeff(6 downto 0);--the coeff only has 7 bits

coeff_2 <= data_coeff(14 downto 8);

if counter_8 = "11" then

counter_8_nxt <= (others => '0');

store_nxt <= "1";--signal to store

if counter_14 = "1101" then

counter_14_nxt <= (others => '0');

state_nxt <= s_send;

else

counter_14_nxt <= counter_14 + 1;

state_nxt <= s_multi;

end if;

else

store_nxt <= "0";--signal to store

counter_14_nxt <= counter_14;

counter_8_nxt <= counter_8 + 1;

state_nxt <= s_multi;

end if;

when s_send =>

multi_done <= '1';--feedback to controller & average

state_nxt <= s_initial;

end case;

end process;

--Computing part with two multipliers-----------------------

result_1 <= input_1 * coeff_1 + "000000000000000000"; --to make they have the same digits

result_2 <= input_2 * coeff_2 + "000000000000000000";

--Send data-------------------------------------------------

store_en <= store(0);

max_en <= store(0);

data_out <= data when store = "1" else (others => '0');

--Flip Flop-------------------------------------------------

counter1: FF

generic map(N => 2)

port map(   D     =>counter_8_nxt,

Q     =>counter_8,

clk     =>clk,

reset   =>reset

);

counter2: FF

generic map(N => 4)

port map(   D     =>counter_14_nxt,

Q     =>counter_14,

clk     =>clk,

reset   =>reset

);

counter3: FF

generic map(N => 3)

port map(   D     =>counter_6_nxt,

Q     =>counter_6,

clk     =>clk,

reset   =>reset

);

add: FF

generic map(N => 18)

port map(   D     =>data_nxt,

Q     =>data,

clk     =>clk,

reset   =>reset

);

send_data: FF

generic map(N => 1)

port map(   D     =>store_nxt,

Q     =>store,

clk     =>clk,

reset   =>reset

);

end Behavioral;

## 4.5 VHDL code for store

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity store is

Port (

clk, reset  : in std_logic;

store_en    : in std_logic;

data_out    : in std_logic_vector(17 downto 0)

);

end store;

architecture Behavioral of store is

component SRAM_store

port (

clk     : in  std_logic;            --Active Low

we      : in std_logic;

a       : in  std_logic_vector (8 downto 0);

d       : in  std_logic_vector (31 downto 0);

qspo    : out std_logic_vector (31 downto 0)

);

end component;

component ff is

generic(N:integer:=1);

port(   D  :  in std_logic_vector(N-1 downto 0);

Q  :  out std_logic_vector(N-1 downto 0);

clk  :  in std_logic;

reset:  in std_logic

);

end component;

--SRAM---------------------------------------------

signal choose       : std_logic;

signal r_or_w       : std_logic;

signal address      : std_logic_vector(8 downto 0) := (others => '0');

signal RY_ram       : std_logic;

signal sram_out     : std_logic_vector(31 downto 0);

---------------------------------------------------

signal address_nxt  : std_logic_vector(8 downto 0);

signal store_data_32 : std_logic_vector(31 downto 0);

begin

--SRAM bits transfer----------------------

store_data_32 <= "00000000000000" & data_out;

------------------------------------------

Ram_store: SRAM_store

port map(

clk     => clk,

we      => r_or_w,

a       => address,

d       => store_data_32,

qspo    => sram_out

);

address_nxt <= address + 1 when store_en = '1' else address;

r_or_w <= '1' when store_en = '1' else '0';

choose <= '0';

data_address: FF

generic map(N => 9)

port map(   D     =>address_nxt,

Q     =>address,

clk     =>clk,

reset   =>reset

);

end Behavioral;

## 4.6 VHDL code for max

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity max is

Port (

clk, reset  : in std_logic;

-----------------------------------------------------

--control signal from multiply

max_en      : in std_logic;

--data from multiply

data_out    : in std_logic_vector(17 downto 0);

-----------------------------------------------------

--find the maximum data

max_out     : out std_logic_vector(17 downto 0);

clk_out     : out std_logic

-----------------------------------------------------

);

end max;

architecture Behavioral of max is

component ff is

generic(N:integer:=1);

port(   D  :  in std_logic_vector(N-1 downto 0);

Q  :  out std_logic_vector(N-1 downto 0);

clk  :  in std_logic;

reset:  in std_logic

);

end component;

signal count, count_nxt : std_logic_vector(6 downto 0);

signal data_max, data_max_nxt   : std_logic_vector(17 downto 0) := (others => '0');

begin

max_out <= data_max;

clk_out <= clk;

data_max_nxt <= data_out when data_max < data_out else data_max;

--Flip Flop-------------------------------------------------

compare_data: FF

generic map(N => 18)

port map(   D     =>data_max_nxt,

Q     =>data_max,

clk     =>clk,

reset   =>reset

);

end Behavioral;

## 4.7 VHDL code for top

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity top is

Port (

--input

clki            : in std_logic;

reseti          : in std_logic;

starti          : in std_logic;

--output

start_ld_inputo : out std_logic;

start_ld_coeffo : out std_logic;

max_outo        : out std_logic_vector(17 downto 0);

clk_out         : out std_logic

);

end top;

architecture Behavioral of top is

component controller is

Port (

clk, reset  : in std_logic;

start       : in std_logic;

ld_input_done   : in std_logic;

multi_done  : in std_logic;

ld_input    : out std_logic;

op_en       : out std_logic

);

end component;

component load_coeff is

Port (

clk, reset          : in std_logic;

op_en               : in std_logic;

multi_en            : out std_logic;

data_coeff          : out std_logic_vector(15 downto 0)

);

end component;

component load_input is

Port (

clk, reset  : in std_logic;

ld_input    : in std_logic;

ld_input_done   : out std_logic;--feedback to controller

op_en       : in std_logic;

start_ld_input  : out std_logic;

data_input  : out std_logic_vector(15 downto 0)

);

end component;

component multiply is

Port (

clk, reset  : in std_logic;

multi_en    : in std_logic;

data_input  : in std_logic_vector(15 downto 0);

data_coeff  : in std_logic_vector(15 downto 0);

multi_done  : out std_logic;

store_en    : out std_logic;

max_en      : out std_logic;

data_out    : out std_logic_vector(17 downto 0)

);

end component;

component store is

Port (

clk, reset  : in std_logic;

store_en    : in std_logic;

data_out    : in std_logic_vector(17 downto 0)

);

end component;

component max is

Port (

clk, reset  : in std_logic;

max_en      : in std_logic;

data_out    : in std_logic_vector(17 downto 0);

max_out     : out std_logic_vector(17 downto 0);

clk_out     : out std_logic

);

end component;

--controller

signal ld_input_done    : std_logic;

signal multi_done       : std_logic;

signal ld_input         : std_logic;

signal op_en            : std_logic;

--ld_coeff

signal multi_en            :  std_logic;

signal data_coeff          :  std_logic_vector(15 downto 0);

--ld_input

signal data_input      :  std_logic_vector(15 downto 0);

--multiply

signal store_en    :  std_logic;

signal data_out    :  std_logic_vector(17 downto 0);

--store

--max

signal max_en      : std_logic;

---------------------------------------------------------

begin

--clk_out <= clki;

controller_part: controller

port map(

clk           => clki          ,

reset         => reseti         ,

start         => starti        ,

ld_input_done => ld_input_done  ,

multi_done    => multi_done    ,

ld_input      => ld_input      ,

op_en         => op_en

);

coeff_part: load_coeff

port map(

clk             => clki          ,

reset           => reseti         ,

op_en           => op_en        ,

multi_en        => multi_en     ,

data_coeff      => data_coeff

);

input_part: load_input

port map(

clk             => clki          ,

reset           => reseti         ,

ld_input        => ld_input     ,

ld_input_done   => ld_input_done  ,

op_en           => op_en        ,

start_ld_input  => start_ld_inputo    ,

data_input      => data_input

);

multiply_part: multiply

port map(

clk        => clki,

reset      => reseti,

multi_en   => multi_en,

data_input => data_input,

data_coeff => data_coeff,

multi_done => multi_done,

store_en => store_en,

max_en   => max_en,

data_out   => data_out

);

store_part: store

port map(

clk        => clki,

reset      => reseti,

store_en   => store_en,

data_out   => data_out

);

max_part: max

port map(

clk      => clki,

reset    => reseti,

max_en   => max_en,

data_out => data_out,

max_out  => max_outo,

clk_out  => clk_out

);

end Behavioral;